AI-BASED SMART PARKING SPACE DETECTION AND RESERVATION SYSTEM

A CAPSTONE PROJECT REPORT

A Capstone Project Report submitted in partial fulfilment of the requirements for the Course of

ITA0609 – MACHINE LEARNING

to the award of the degree of

BACHELOR OF ENGINEERING

IN

ELECTRONICS AND COMMUNICATION ENGINEERING

(ECE)

Submitted by

K. Vishwanathan (192312063)

Under the Supervision of

Dr. Shri Vindhya

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

SEPTEMBER 2026

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

BONAFIDE CERTIFICATE

This is to certify that the Capstone Project entitled “AI-Based Smart Parking Space Detection and Reservation System” has been carried out by K. Vishwanathan (Reg. No. 192312063) under the supervision of Dr. Shri Vindhya and is submitted in partial fulfilment of the requirements for the current semester of the B.E Electronics and Communication Engineering program at Saveetha Institute of Medical and Technical Sciences, Chennai.

| SIGNATURE COURSE COORDINATOR Dr. T J Nagalakshmi, Program Director, Electronics and Communication Engineering, SIMATS Engineering, SIMATS, Chennai – 602105. | SIGNATURE COURSE FACULTY Dr. Shri Vindhya, Professor, Department of Machine Learning, SIMATS Engineering, SIMATS, Chennai – 602105. |
| --- | --- |

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

DECLARATION

I, K. Vishwanathan of the Electronics and Communication Engineering Department, SIMATS Engineering, Saveetha Institute of Medical and Technical Sciences, Chennai, hereby declare that the Capstone Project Work entitled “AI-Based Smart Parking Space Detection and Reservation System” is the result of my own bonafide efforts. To the best of my knowledge, the work presented herein is original, accurate, and has been carried out in accordance with the principles of engineering ethics and academic integrity. All sources, references, data, and other materials used in the project have been appropriately acknowledged. I further declare that the data, results, analysis, and findings presented in this report have not been fabricated, falsified, or manipulated. Any external tools, software, datasets, computational resources, or AI-assisted tools used during the project have been appropriately disclosed. I confirm that all applicable ethical, academic, institutional, and professional requirements have been duly followed throughout the planning, execution, analysis, and documentation of the project.

Place: ______________________

Date: ______________________

Signature of the Student with Name

K. Vishwanathan (192312063)

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

Individual Contribution Statement

| Student Name / Register No. | Specific Responsibilities | Design & Development Contribution | Testing & Analysis Contribution | Report Contribution | Approx. Contribution (%) |
| --- | --- | --- | --- | --- | --- |
| K. Vishwanathan 192312063 | Module 2 – Reservation Slot: reservation logic, slot booking workflow, and dataset handling/preprocessing for the reservation module | Designed and implemented the slot reservation module, including slot selection, booking, and confirmation-status logic | Performed preprocessing checks and functional testing of the reservation workflow | Report sections covering the methodology and implementation of the Reservation Slot module (Module 2) | 100% |
| Total |  |  |  |  | 100 % |

ABSTRACT

Urban parking facilities face increasing pressure due to rising vehicle density, limited parking capacity, and the absence of reliable real-time occupancy information. Drivers often spend considerable time searching for vacant spaces, resulting in unnecessary vehicle movement, fuel consumption, congestion, and inconvenience. As part of a three-member capstone team, I contributed to an AI-Based Smart Parking Space Detection and Reservation System that combines computer vision, machine learning, and a digital reservation mechanism to improve parking-space utilisation and user convenience. My individually assigned responsibility within the project was Module 2 – Reservation Slot, covering dataset handling, preprocessing, and the design and implementation of the slot reservation logic.

The overall system uses parking-lot image data and computer-vision processing to identify individual parking slots and classify them as vacant or occupied; image preprocessing includes resizing, normalisation, noise handling, and slot-based cropping. Within this system, the Reservation Slot module I developed reads the current occupancy state produced by the detection engine, allows a user to select an available slot, and confirms or rejects the booking based on the slot's real-time reservation state, preventing duplicate reservation of the same slot. A parking-management dashboard, developed by another team member, presents occupied, vacant, and reserved spaces together with basic occupancy and reservation analytics.

The implementation is based on Python. My module uses Pandas and NumPy for reservation-state and slot-data handling, and integrates with the OpenCV-based detection pipeline and the Scikit-learn/TensorFlow-Keras classification model developed by the team. Testing of my module focused on functional verification of slot selection, booking confirmation, and double-booking prevention, in addition to preprocessing checks on the reservation dataset. The prototype as a whole demonstrates the feasibility of combining AI-based parking detection with reservation and management functions in a single system. Major limitations include sensitivity to lighting and camera position for the detection stage, dataset diversity, and the need for stronger concurrency handling in the reservation logic under multi-user conditions.

The project concludes that an integrated machine-learning smart-parking approach, with a dedicated and independently verifiable reservation module, provides a practical foundation for intelligent parking management. Future enhancements identified for the reservation module include stronger concurrency control, user authentication, integration with automated payments, and cloud-based synchronisation across multiple parking facilities.

Keywords: Smart Parking, Machine Learning, Reservation System, Slot Booking, Computer Vision, Parking Occupancy Detection

TABLE OF CONTENTS

| Sl. No. | Title | Page No. |
| --- | --- | --- |
| i | CERTIFICATE FROM THE SUPERVISOR | ii |
| ii | DECLARATION BY THE CANDIDATE | iii |
| iii | INDIVIDUAL CONTRIBUTION STATEMENT | iv |
| iv | ABSTRACT | v |
| v | LIST OF FIGURES | vi |
| vi | LIST OF TABLES | vii |
| vii | LIST OF ABBREVIATIONS | viii |
| 1 | CHAPTER 1: Introduction and Engineering Problem | 1 |
|  | 1.1 Background | 1 |
|  | 1.2 Need for the Project | 1 |
|  | 1.3 Problem Statement | 1 |
|  | 1.4 Project Aim | 2 |
|  | 1.5 Project Objectives | 2 |
|  | 1.6 Scope | 2 |
|  | 1.7 Expected Engineering Outcome | 2 |
| 2 | CHAPTER 2: Literature Review and Existing Solutions | 3 |
|  | 2.1 Review Method | 3 |
|  | 2.2 Review of Existing Work | 3 |
|  | 2.3 Comparative Analysis | 3 |
|  | 2.4 Research / Engineering Gap | 3 |
|  | 2.5 Proposed Contribution | 4 |
| 3 | CHAPTER 3: Engineering Design & Trade-offs, Sustainability, Ethics, Safety, Risk & Security | 4 |
|  | 3.1 Requirements | 4 |
|  | 3.2 Constraints & Applicable Standards | 5 |
|  | 3.3 Design Specifications | 5 |
|  | 3.4 Alternative Solutions & Evaluation | 6 |
|  | 3.5 Selected Design & Detailed Design | 6 |
|  | 3.6 Trade-off Analysis | 7 |
|  | 3.7 Sustainability, Ethics, Safety, Risk & Security | 7 |
| 4 | CHAPTER 4: Implementation, Testing & Results, Project Management | 8 |
|  | 4.1 Development Approach & Resources | 8 |
|  | 4.2 Software Implementation & Modern Tools Used | 8 |
|  | 4.3 Test Plan & Verification | 9 |
|  | 4.4 Results | 9 |
|  | 4.5 Data Analysis & Engineering Judgment | 10 |
|  | 4.6 Comparison with Existing Solutions & Achievement of Objectives | 10 |
|  | 4.7 Strengths & Limitations | 10 |
|  | 4.8 Project Planning, Roles & Budget | 11 |
| 5 | CHAPTER 5: Conclusion, Future Work and Reflection | 12 |
|  | 5.1 Conclusion | 12 |
|  | 5.2 Future Work | 12 |
|  | 5.3 Professional Learning & Individual Reflection | 12 |
|  | References | 13 |
|  | Appendices | 14 |

LIST OF FIGURES

| Figure No. | Figure Name | Page No. |
| --- | --- | --- |
| Figure 1 | System Architecture – Detection, Reservation & Dashboard Flow | 17 |
| Figure 2 | Reservation Module – Slot Selection Workflow | 17 |
| Figure 3 | Reservation Confirmation / Double-Booking Prevention Flow | 21 |
| Figure 4 | Parking Slot Reservation Interface (Dashboard View) | Appendix / Results |

LIST OF TABLES

| Table No. | Table Name | Page No. |
| --- | --- | --- |
| Table 1 | Stakeholder and User Requirements | Chapter 3 |
| Table 2 | Functional and Non-Functional Requirements | Chapter 3 |
| Table 3 | Applicable Standards | Chapter 3 |
| Table 4 | Design Specifications | Chapter 3 |
| Table 5 | Alternative Solution Evaluation | Chapter 3 |
| Table 6 | Trade-off Analysis | Chapter 3 |
| Table 7 | Sustainability, Ethics, Safety and Security | Chapter 3 |
| Table 8 | Risk Register | Chapter 3 |
| Table 9 | Test Plan and Verification (Reservation Module) | Chapter 4 |
| Table 10 | Results Summary | Chapter 4 |
| Table 11 | Objective Achievement | Chapter 4 |
| Table 12 | Project Roles and Contribution | Chapter 4 |
| Table 13 | Estimated Project Budget | Chapter 4 |

LIST OF ABBREVIATIONS / SYMBOLS

| Symbol | Description | Unit |
| --- | --- | --- |
| AI | Artificial Intelligence | — |
| ML | Machine Learning | — |
| CV | Computer Vision | — |
| CNN | Convolutional Neural Network | — |
| UI | User Interface | — |
| SRS | Software Requirements Specification | — |
| IEEE | Institute of Electrical and Electronics Engineers | — |
| ISO/IEC | International Organization for Standardization / International Electrotechnical Commission | — |

CHAPTER 1

INTRODUCTION AND ENGINEERING PROBLEM

1.1 Background

Urban areas are experiencing a rapid increase in vehicle density, creating parking shortages and traffic congestion around commercial, residential, institutional, and public facilities. Drivers frequently spend time searching for available spaces, increasing fuel consumption, emissions, and frustration. Traditional parking systems based on manual monitoring, static signage, or simple counters often do not provide detailed real-time information for individual slots.

Artificial Intelligence, Machine Learning, and Computer Vision provide an engineering approach for converting parking-lot images into useful occupancy information. As part of a three-member capstone team, this project addresses the problem by detecting individual spaces, classifying their status, supporting digital reservation, and presenting the resulting information through a management dashboard. My individually assigned responsibility within this system is Module 2 – Reservation Slot: turning the detected occupancy state into a reliable, conflict-free slot reservation workflow.

1.2 Need for the Project

- Real-time parking occupancy information is required to reduce unnecessary searching and vehicle circulation.

- Drivers need a convenient method to identify and reserve suitable vacant spaces without double-booking.

- Parking operators need centralized information about occupancy, reservations, and utilization.

- Camera-based automation can reduce dependence on continuous manual monitoring.

- A dependable reservation layer is required so that a vacant slot detected by the system can be confidently booked by only one user at a time.

1.3 Problem Statement

The engineering problem is to develop a reliable and practical smart parking system that can determine the occupancy status of individual parking slots from parking-lot image data, update availability information, and support reservation without double-booking. Within this system, the specific problem addressed by my module is to design a reservation mechanism that reads the current occupancy/availability state, allows a user to select and book a vacant slot, validates the request against the slot's existing reservation state, and returns a clear confirmation or rejection – all while remaining consistent with the detection output and simple enough to operate under prototype-level computational constraints.

1.4 Project Aim

To design and implement an AI-based smart parking prototype that detects vacant and occupied parking spaces using machine-learning and computer-vision techniques, and to design, implement and test a slot reservation module (Module 2) that converts this detection output into a dependable, conflict-free booking workflow integrated with the parking management dashboard.

1.5 Project Objectives

- To analyze parking-lot image data and the associated slot/occupancy dataset used by the reservation module.

- To prepare and preprocess reservation-related data (slot identifiers, occupancy state, reservation status) for use by the booking logic.

- To design a reservation data structure that records each slot's availability and reservation status.

- To implement slot selection and booking logic that checks availability before confirming a reservation.

- To implement double-booking prevention so an already reserved slot cannot be reserved again.

- To test the reservation workflow functionally and verify correct behaviour under normal and conflicting booking requests.

1.6 Scope

Included in my module: reservation-data handling and preprocessing, design of the slot reservation/booking logic, confirmation and rejection messaging, and functional testing of the reservation workflow, integrated with the occupancy-detection output produced by the team's detection module and displayed through the team's dashboard module.

Excluded from my module (handled elsewhere in the system or outside the current prototype): the computer-vision occupancy-classification model itself, dashboard visual design, production-grade payment gateways, user authentication and account management, large-scale cloud deployment, and guaranteed performance under every possible concurrent-user load.

Assumptions include that the detection module supplies a valid vacant/occupied status for each configured slot, and that reservation requests are processed one at a time in the prototype environment. Boundary conditions include the number of configured slots, the format of the incoming occupancy data, and the single-session nature of the prototype's booking logic.

1.7 Expected Engineering Outcome

The expected outcome of my module is a working reservation sub-system – a set of data structures and functions that record slot availability, accept a reservation request, validate it against the current reservation state, and return a confirmation or a “slot already reserved” rejection. Integrated with the detection engine and dashboard developed by my teammates, this produces a complete prototype in which a user can view available slots and reserve one without conflict.

CHAPTER 2

LITERATURE REVIEW AND EXISTING SOLUTIONS

2.1 Review Method

The review was structured around the technical areas relevant to my module and the overall system: machine-learning based parking occupancy detection, computer vision for parking-slot analysis, and – with particular attention for Module 2 – digital parking-reservation systems and conflict/concurrency handling in booking workflows. The project team's reference list was used as the basis for identifying relevant approaches and limitations.

2.2 Review of Existing Work

Existing smart-parking research commonly uses image classification, object detection, and sensor-based occupancy detection to determine whether a slot is vacant or occupied. Camera-based systems can provide detailed slot-level information, while IoT and ultrasonic sensors provide direct occupancy measurement. Reservation-enabled systems add a user-facing decision and booking layer on top of this detection output; the main engineering challenge in this layer, relevant to my module, is ensuring that two users cannot simultaneously reserve the same slot and that the reservation state stays consistent with the underlying occupancy data.

| Existing Approach | Advantages | Limitations | Relevance to Proposed Work |
| --- | --- | --- | --- |
| Manual monitoring / static signage | Simple to deploy | Not real-time at individual-slot level; no reservation capability | Motivates automated occupancy detection and digital reservation |
| Camera + image classification | Detailed visual slot information | Sensitive to lighting, viewpoint and image quality | Supplies the occupancy state that my reservation module consumes |
| Sensor-based occupancy detection | Direct occupancy measurement; useful in difficult visual conditions | Requires hardware installation and maintenance | Potential future input source for the reservation module |
| Reservation-enabled smart parking (booking apps) | Improves convenience and planning; prevents duplicate booking | Requires conflict handling and synchronized availability data | Directly comparable to Module 2 – the reservation logic implemented here |

2.3 Comparative Analysis

The proposed system combines slot-level computer-vision detection with a reservation module and a dashboard. This integration distinguishes it from approaches that focus only on detection or only on booking. My module specifically addresses the booking layer: given a slot's detected/administered status, it determines whether a reservation can be confirmed, which is the aspect most directly comparable to existing reservation-enabled parking applications.

2.4 Research / Engineering Gap

A practical gap remains between occupancy-detection research and a simple, reliable workflow that connects detected slot status directly to reservation and management visualization. In particular, concurrent or near-simultaneous booking requests, and keeping reservation state synchronized with detection output, remain important for reliable deployment – this is the specific gap that Module 2 (Reservation Slot) is intended to address within the prototype.

2.5 Proposed Contribution

My contribution within the team project is a reservation module that reads slot availability, applies a simple but robust check-before-confirm booking rule, and prevents duplicate reservation of an already-booked slot. Combined with the detection module and dashboard developed by my teammates, this produces an integrated prototype linking parking image preprocessing, slot occupancy classification, reservation management, and dashboard visualization.

CHAPTER 3

ENGINEERING DESIGN & TRADE-OFFS, SUSTAINABILITY, ETHICS, SAFETY, RISK & SECURITY

3.1 Requirements

Stakeholder / User Requirements (Table 1)

| Stakeholder | Requirement |
| --- | --- |
| Drivers | View current parking availability and reserve a suitable vacant slot without conflict. |
| Parking Operators | Monitor occupancy, reservations, utilization, and parking trends. |
| Facility Management | Obtain centralized parking information and usage reports. |
| Project Developers / Researchers | Evaluate reservation-workflow correctness and extend the AI-based system. |

Functional & Non-Functional Requirements (Table 2)

| Requirement Type | Requirement | Measurable Target |
| --- | --- | --- |
| Functional | Detect slot occupancy (team module) | Output vacant/occupied status for configured slots |
| Functional | Reserve a slot (Module 2) | Prevent reservation of an already reserved slot |
| Functional | Release / query reservation state (Module 2) | Correctly report current reservation status per slot |
| Functional | Display dashboard | Show availability and reservation information |
| Non-Functional | Usability | Simple interactive reservation workflow |
| Non-Functional | Reliability | Consistent reservation outcome for the same input conditions |
| Non-Functional | Maintainability | Modular Python implementation of the reservation logic |
| Non-Functional | Scalability | Allow additional slots and future concurrency/authentication features |

3.2 Constraints & Applicable Standards (Table 3)

| Standard / Code | Requirement | How Applied in This Project |
| --- | --- | --- |
| IEEE 12207 | Software life-cycle processes | Used as a reference for systematic development, testing, and maintenance of the reservation module |
| IEEE 830 | Software Requirements Specification | Used as a reference for structuring the reservation module's functional and non-functional requirements |
| ISO/IEC 25010 | Software quality model | Used to consider reliability, usability and maintainability of the reservation logic |
| IEEE 1012 | Software verification and validation | Used as a reference for functional verification and testing of the reservation workflow |

Key constraints on my module: the reservation logic depends on receiving a valid occupancy status from the detection module; it must process booking requests one at a time in the prototype to avoid race conditions; and it must keep reservation and user-identifier data minimal for privacy and security.

3.3 Design Specifications (Table 4)

| Parameter | Target/Requirement | Unit | Priority |
| --- | --- | --- | --- |
| Reservation state | Available / Reserved | state | High |
| Slot identifier | Unique slot ID mapped to detection output | id | High |
| Booking request handling | Check-before-confirm | logic | High |
| Reservation data storage | In-memory dictionary (prototype) | structure | Medium |
| Dashboard integration | Reservation status feed to dashboard | interface | Medium |

3.4 Alternative Solutions & Evaluation

Alternative 1: In-memory dictionary-based reservation store (selected). Each slot ID is mapped to a reservation status; a booking request is confirmed only if the slot is not already reserved. Simple, fast, and adequate for a single-session prototype.

Alternative 2: File/CSV-based reservation log. Reservation entries are appended to a file and checked before confirming a new booking. More persistent than an in-memory store but slower and more error-prone for concurrent access.

Alternative 3: Database-backed reservation table with transaction locking. Would provide strong concurrency control and persistence, appropriate for production deployment, but adds infrastructure complexity beyond the current prototype scope.

| Criterion | Weight | Alt. 1 In-Memory | Alt. 2 File/CSV | Alt. 3 Database |
| --- | --- | --- | --- | --- |
| Performance | High | Good | Moderate | Good |
| Cost | High | Low | Low | Moderate/High |
| Safety (double-booking) | High | Good (single-session) | Moderate | Very Good |
| Sustainability | Medium | Good | Good | Moderate |
| Reliability | High | Good for prototype | Moderate | Very Good |

3.5 Selected Design & Detailed Design

The in-memory, check-before-confirm reservation design (Alternative 1) was selected because it matches the prototype's software-oriented scope, is simple to test and demonstrate, and directly satisfies the double-booking-prevention requirement without adding database infrastructure. Justification follows directly from the evaluation matrix in Section 3.4, where Alternative 1 scores well on performance, cost, and reliability for a single-session prototype, with database-backed storage (Alternative 3) identified as the appropriate upgrade path for production deployment.

Module 2 data/control flow: Detected occupancy status (from the team's detection module) → slot availability map → user selects a vacant slot → reservation request → check current reservation state → if free: confirm and mark slot as Reserved; if already reserved: return “Slot Already Reserved” → updated status pushed to the dashboard.

Key relationship used in the reservation logic: a slot is bookable only when detection_status = “Vacant” AND reservation_status = “Available”; confirming a booking sets reservation_status = “Reserved” for that slot ID.

Systems approach: the reservation module depends on the detection module for the current vacant/occupied signal and supplies its own Available/Reserved state to the dashboard module, so the three subsystems interface through a shared slot-identifier scheme; a change in slot numbering in the detection module directly affects the reservation module's lookups, which is why a common slot-coordinate/ID convention was agreed with the rest of the team.

3.6 Trade-off Analysis (Table 6)

| Design Decision | Alternative Considered | Benefit | Disadvantage | Final Decision & Justification |
| --- | --- | --- | --- | --- |
| Reservation storage | Database with locking | Lower complexity, faster to build and test | No persistence across restarts | Selected in-memory store for prototype; database noted as future work |
| Booking rule | Optimistic booking with later reconciliation | Simple to implement | Risk of double-booking under concurrency | Selected check-before-confirm rule to directly prevent conflicts |
| Slot identification | Auto-discovered coordinates | No manual mapping needed | Requires reliable auto-detection | Selected predefined slot IDs shared with detection module for reliable testing |

3.7 Sustainability, Ethics, Safety, Risk & Security (Table 7)

| Domain | Consideration | Evidence Evaluated | Impact on Final Decision |
| --- | --- | --- | --- |
| Sustainability | Reduce unnecessary vehicle circulation while searching for parking | Reservation confirms a specific slot in advance, so a driver does not need to search once a booking is confirmed | Supported keeping the reservation confirmation immediate and unambiguous |
| Ethics | Responsible handling of user/slot data in the reservation record | Reservation record stores only a slot ID and minimal user reference, not full personal profiles | Avoided storing unnecessary personal data in the reservation structure |
| Health & Safety | Reduce in-facility searching and congestion caused by uncertain availability | Reservation status is checked before confirming, avoiding drivers being directed to an already-taken slot | Reinforced the check-before-confirm design rule |
| Security | Prevent unauthorized or conflicting reservations of the same slot | Reservation state is checked and updated atomically per request in the prototype loop | Directly shaped the core booking-validation logic of reservation module |

Risk Register (Table 8)

| Risk | Likelihood | Severity | Risk Level | Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Reservation conflict / double-booking | Medium | High | High | Check reservation state before confirming any booking | Low/Medium |
| Reservation state out of sync with detection output | Medium | Medium | Medium | Shared slot-ID convention with the detection module | Medium |
| Loss of reservation data (no persistence in prototype) | Medium | Medium | Medium | Documented as a future-work item for database-backed storage | Medium |
| Unauthorized access to reservation/user data | Low/Medium | High | Medium/High | Minimal data collection; access control in future deployment | Medium |

CHAPTER 4

IMPLEMENTATION, TESTING & RESULTS, PROJECT MANAGEMENT

4.1 Development Approach & Resources

My development of Module 2 followed a modular workflow: understanding the slot/occupancy data format produced by the detection module, preparing and preprocessing the reservation dataset, designing the reservation data structure, implementing the booking and conflict-check logic, and testing it against normal and conflicting booking requests. The module was built so it could be developed and tested independently of the detection and dashboard modules, using sample slot-status data, and then integrated with my teammates' components.

| Resource | Purpose |
| --- | --- |
| Python | Implementation language for the reservation module |
| Pandas / NumPy | Handling and preprocessing of slot/reservation data |
| Parking slot data (slot IDs, coordinates) | Shared reference used to key reservation records to detection output |
| Occupancy status feed (from detection module) | Input signal indicating which slots are currently vacant |
| Streamlit (integration point) | Where reservation status is displayed to the user, in the team dashboard |

4.2 Software Implementation & Modern Tools Used

Reservation Module (Module 2) – Implementation Details

The reservation module maintains a dictionary keyed by slot ID, storing the current reservation status of every slot. When a user requests a slot, the module first checks the detection module's reported status for that slot and its own reservation dictionary; the request is confirmed only if the slot is both physically vacant and not already reserved. On confirmation, the reservation dictionary is updated and the new status is made available to the dashboard.

| Tool / Software | Purpose | Stage Used |
| --- | --- | --- |
| Python | Core implementation of reservation logic | All stages of Module 2 |
| Pandas | Loading and filtering slot/reservation data | Preprocessing |
| NumPy | Basic numerical/array handling for slot indices | Preprocessing |
| Dictionary-based in-memory store | Reservation status storage | Implementation |
| Streamlit (shared with dashboard module) | Displaying reservation confirmation to the user | Integration |

4.3 Test Plan & Verification (Table 9)

| Requirement | Test Method | Acceptance Criterion | Achieved Value | Status |
| --- | --- | --- | --- | --- |
| Reserve an available slot | Request reservation on a vacant, unreserved slot | Confirmation message returned | Confirmed | Pass |
| Prevent double-booking | Request reservation on an already-reserved slot | “Slot Already Reserved” returned; original reservation retained | Blocked as expected | Pass |
| Reject reservation on occupied slot | Request reservation on a slot detected as Occupied | Request rejected | Rejected as expected | Pass |
| Reservation status query | Query current status of a given slot ID | Correct Available/Reserved value returned | Correct value returned | Pass |
| Data preprocessing check | Load and validate sample slot/reservation dataset | No missing/invalid slot IDs after preprocessing | Dataset validated | Pass |

4.4 Results (Table 10)

The reservation module successfully demonstrated its core function: a user could select a vacant slot and receive a booking confirmation, while a repeated request on the same slot correctly returned a rejection. Preprocessing checks on the sample slot/reservation dataset did not reveal missing or invalid slot identifiers. Because Module 2 is a logic-and-data module rather than a classification model, its evaluation is functional (pass/fail against the test plan in Table 9) rather than accuracy-based; the occupancy-classification accuracy itself is produced by the detection module developed by another team member.

| Result Area | Observed Outcome |
| --- | --- |
| Slot Reservation | Booking confirmation generated for available slots |
| Double-Booking Prevention | Second reservation attempt on the same slot correctly blocked |
| Reservation Status Query | Correct Available/Reserved status returned for queried slots |
| Data Preprocessing | Sample reservation dataset validated with no invalid slot IDs |
| Dashboard Integration | Reservation status correctly reflected in the shared dashboard view |

4.5 Data Analysis & Engineering Judgment

Functional testing of Module 2 showed that the check-before-confirm rule reliably prevents duplicate reservation of the same slot within a single-session prototype. The main engineering limitation identified is that the in-memory reservation store does not persist across restarts and has not been tested under genuinely concurrent multi-user requests, so its robustness under simultaneous booking attempts from many users is not yet established. This indicates that, before any real deployment, the reservation store should be migrated to a database with transaction-level locking, as identified in Section 3.4/3.6.

4.6 Comparison with Existing Solutions & Achievement of Objectives (Table 11)

| Objective | Evidence | Achievement |
| --- | --- | --- |
| Prepare and preprocess reservation-related data | Slot/reservation dataset loaded and validated with Pandas/NumPy | Fully Achieved |
| Design a reservation data structure | Slot-ID keyed reservation dictionary implemented | Fully Achieved |
| Implement slot selection and booking logic | Reserve-slot function implemented and tested | Fully Achieved |
| Implement double-booking prevention | Verified via functional test cases (Table 9) | Fully Achieved |
| Integrate with detection output and dashboard | Reservation status keyed to shared slot IDs and surfaced on dashboard | Fully Achieved |
| Validate under concurrent multi-user booking load | Not tested in the current single-session prototype | Partially Achieved |

4.7 Strengths & Limitations

Strengths

- Reliable check-before-confirm booking logic that prevents duplicate reservation of the same slot.

- Simple, modular Python implementation that is easy to test independently of the detection and dashboard modules.

- Clean integration point with the detection module through a shared slot-ID convention.

- Minimal data collection in the reservation record, supporting the privacy considerations in Section 3.7.

Limitations

- The in-memory reservation store does not persist reservation data across restarts.

- Concurrency handling for many simultaneous booking requests has not been tested in the current prototype.

- The module depends entirely on receiving a correct occupancy status from the detection module.

- User authentication and production-level security controls are outside the current prototype scope.

4.8 Project Planning, Roles & Budget

| Milestone | Planned Activity | Outcome |
| --- | --- | --- |
| 1 | Problem identification and requirement analysis | Problem statement and objectives defined for the whole system, including reservation slot module |
| 2 | Dataset and methodology preparation | Parking and reservation data workflow prepared |
| 3 | Detection module development (teammate) | Occupancy classification prototype |
| 4 | Reservation module development (my module) | Slot booking logic with double-booking prevention |
| 5 | Dashboard integration (teammate) | Interactive management interface |
| 6 | Testing and evaluation | Functional testing of reservation module and accuracy evaluation of detection module |
| 7 | Documentation and final review | Capstone report and presentation |

Project Roles and Contribution (Table 12)

| Student | Assigned Responsibility | Actual Contribution |
| --- | --- | --- |
| S. Swagat | System integration, detection workflow, report coordination | Detection/integration/testing/report |
| K. Vishwanathan (myself) | Data preparation, preprocessing, reservation module | Dataset handling, preprocessing, reservation logic and testing |
| G. Vamsi | Dashboard, visualization, evaluation and documentation | Dashboard/results/documentation |

### Fig 1 : MODULE 1: Parking Space Detection

Estimated Project Budget

| Item | Estimated Cost (INR) | Actual Cost (INR) |
| --- | --- | --- |
| Software and development tools | 0–1,000 | 0–1,000 |
| Dataset / digital resources | 0–500 | 0–500 |
| Optional computing / cloud resources | 0–1,000 | 0–1,000 |
| Documentation / presentation | 500 | 500 |
| Total | Up to approximately 3,000 | Within project budget |

### Fig 2 : MODULE 2: Parking Slot Reservation

Fig 3 : MODULE 3: Parking Management Dashboard

CHAPTER 5

CONCLUSION, FUTURE WORK AND REFLECTION

5.1 Conclusion

The AI-Based Smart Parking Space Detection and Reservation System was developed by our three-member capstone team to address the engineering problem of inefficient parking-space identification and management. Within this system, I was responsible for Module 2 – Reservation Slot: preparing the reservation-related data and designing and implementing the slot booking logic that converts the detection module's occupancy output into a dependable, conflict-free reservation workflow.

Functional testing demonstrated that the reservation module correctly confirms bookings for available slots and reliably prevents double-booking of an already-reserved slot, meeting the objectives set out for Module 2 in Chapter 1. The exercise also highlighted that broader validation – particularly under concurrent multi-user booking requests and with persistent (database-backed) storage – is needed before the reservation module could be considered ready for real-world deployment. Overall, my contribution establishes a practical, tested reservation layer for the team's smart-parking prototype.

5.2 Future Work

- Migrate the reservation store from an in-memory dictionary to a database with transaction-level locking to support persistence and safe concurrency.

- Add user authentication so reservations can be tied to verified individual users.

- Test the reservation module under simulated concurrent multi-user booking load.

- Integrate automated payment confirmation as part of the booking flow.

- Extend the reservation module to support time-bound bookings (reserve for a specific time window) rather than indefinite reservation.

- Synchronize reservation state across multiple parking facilities through a cloud-based backend.

5.3 Professional Learning & Individual Reflection

New Knowledge Acquired

- How a reservation/booking layer should be designed to consume the output of a machine-learning detection pipeline.

- Practical data preprocessing and validation techniques using Pandas and NumPy for slot/reservation datasets.

- Design of check-before-confirm logic to prevent race-condition-style conflicts such as double-booking.

- How to structure a module so it can be developed and tested independently before integration with teammates' components.

- Engineering consideration of standards, risk, safety, sustainability and security as applied to a reservation subsystem.

Engineering & Professional Skills Developed

- Python programming and modular software development.

- Data preprocessing and validation workflow design.

- Functional test-case design and systematic verification against a test plan.

- Team coordination around a shared slot-ID convention and integration interface.

- Engineering decision-making under prototype-level computational and time constraints.

Individual Reflection – K. Vishwanathan (192312063)

A major challenge was organizing the parking and reservation data so that the preprocessing stage produced consistent, valid inputs for the booking logic, and then designing a reservation rule that would reliably prevent double-booking without adding unnecessary complexity to the prototype. I developed a better understanding of how data quality affects downstream logic, and how a reservation module must depend cleanly on the detection module's output through a shared, well-defined slot-identifier convention. A key engineering decision I made personally was to keep the reservation store as a simple, testable in-memory dictionary with a strict check-before-confirm rule, rather than attempting a more complex concurrent design within the prototype's time constraints – a decision supported by the evaluation in Section 3.4. New knowledge I acquired independently included structuring functional test cases specifically to probe conflict scenarios (for example, requesting the same slot twice) rather than only testing the “happy path”. If the module were redesigned, I would move the reservation store to a small database with proper locking and add basic concurrency tests, since this is the limitation most likely to matter in a real deployment.

REFERENCES

[1] Kumar, R., & Sharma, P., “Machine learning based smart parking detection systems for urban infrastructure,” International Journal of Intelligent Transportation Systems, 12(2), 85–96, 2023.

[2] Li, X., & Chen, Y., “Computer vision approaches for real-time parking occupancy detection,” Journal of Artificial Intelligence Research in Transportation, 8(1), 45–58, 2022.

[3] Ahmed, S., & Rao, K., “Image classification techniques for parking slot analysis,” International Journal of Computer Applications in Smart Cities, 10(4), 210–221, 2021.

[4] Patel, M., & Verma, A., “Intelligent parking reservation systems using machine learning,” Advances in Artificial Intelligence and Smart Infrastructure, 14(3), 155–166, 2023.

[5] Wang, T., & Liu, H., “Deep learning models for dynamic parking occupancy prediction,” IEEE Transactions on Smart Transportation Systems, 16(2), 98–109, 2022.

[6] Nair, V., & Joseph, R., “Comparative analysis of machine learning techniques for occupancy detection systems,” International Journal of Sustainable Smart Computing, 7(4), 120–131, 2021.

[7] Singh, P., & Banerjee, D., “AI-based predictive analytics in modern smart city infrastructures,” Journal of Artificial Intelligence and Smart Technology, 9(1), 60–73, 2024.

[8] Zhao, L., & Kim, S., “Data preprocessing methods for intelligent parking prediction applications,” Journal of Computational Urban Systems, 5(3), 180–191, 2022.

[9] Reddy, M., & Thomas, J., “Intelligent parking slot detection using probabilistic and neural models,” Journal of Smart Computing and Transportation Automation, 11(2), 133–145, 2023.

[10] Zhang, Y., & Wu, F., “Application of artificial intelligence in smart parking management systems: A practical approach,” Automation and Intelligent Transportation Studies, 6(4), 201–214, 2021.

[11] IEEE Std 12207, Software Life Cycle Processes.

[12] IEEE Std 830, Software Requirements Specification.

[13] ISO/IEC 25010, Systems and software engineering — Systems and software Quality Requirements and Evaluation (SQuaRE).

[14] IEEE Std 1012, Software Verification and Validation.

APPENDICES

Appendix A – Detailed Calculations

Reservation Slot is a logic-and-data module and does not produce a classification accuracy figure itself; that evaluation belongs to the detection module. The relevant calculation for my module is the functional pass rate against the test plan in Table 9: 5 of 5 defined reservation test cases passed, giving a functional test pass rate of (5/5) × 100 = 100% for the scenarios defined.

Appendix B – Engineering Drawings / Reservation Module Flowchart

Detected Occupancy Status → Slot Availability Map → User Selects Slot → Reservation Request → Check Detection Status & Reservation State → [Available] Confirm Reservation → Update Reservation Dictionary → Push Status to Dashboard → [Already Reserved] Return Rejection Message.

Appendix C – Source Code / Code Repository Information (Reservation Module)

The essential reservation-module logic I implemented is summarized below. Complete source code for the full team prototype is maintained separately in the project repository.

reservations = {}

def reserve_slot(slot_id, detection_status, user_id):

if detection_status.get(slot_id) != "Vacant":

return "Slot Not Available"

if reservations.get(slot_id) is None:

reservations[slot_id] = user_id

return "Reservation Confirmed"

return "Slot Already Reserved"

def release_slot(slot_id):

if slot_id in reservations:

del reservations[slot_id]

return "Slot Released"

return "Slot Was Not Reserved"

def get_reservation_status(slot_id):

return "Reserved" if slot_id in reservations else "Available"

Appendix D – Datasheets

No dedicated hardware datasheet applies to the reservation module, which is a purely software (data and logic) component. The camera/imaging hardware datasheet, where applicable, is documented under the detection module.

Appendix E – Experimental / Raw Data (Reservation Test Scenarios)

| Test Case | Scenario | Expected Result | Observed Result |
| --- | --- | --- | --- |
| TC1 | Reserve a vacant, unreserved slot | Reservation confirmed | Confirmed |
| TC2 | Reserve an already-reserved slot | Rejected – slot already reserved | Rejected as expected |
| TC3 | Reserve a slot marked Occupied by detection | Rejected – slot not available | Rejected as expected |
| TC4 | Query reservation status of a free slot | “Available” returned | Correct |
| TC5 | Query reservation status after booking | “Reserved” returned | Correct |

Appendix F – Ethics / Institutional Approval

The reservation module stores only a slot identifier and a minimal user reference; no sensitive personal data is collected in the current prototype. Before real-world deployment, institutional approval and applicable data-protection requirements for storing user booking information should be reviewed, consistent with the ethics consideration in Section 3.7.

Appendix G – Project Meeting / Progress Records

| Stage | Activity | Evidence / Record |
| --- | --- | --- |
| 1 | Requirement analysis | Project and module-level requirements agreed with team |
| 2 | Design | Reservation data structure and booking-rule design finalized |
| 3 | Implementation | Reservation module developed |
| 4 | Testing | Functional test cases for reservation module executed |
| 5 | Documentation | capstone report prepared |

Appendix H – User / Industry Feedback

No formal user or industry feedback has been collected for the reservation module at this stage. This section can be completed after a faculty or user evaluation session.

Appendix I – Publications / Patents / Competitions / Product Outcomes

No publication, patent, competition or commercial product outcome is associated with the reservation module at this stage.

Appendix J – Individual Contribution Evidence

Student: K. Vishwanathan (192312063). As documented in the Individual Contribution Statement, I was responsible for Module 2 – Reservation Slot, covering dataset handling and preprocessing for the reservation data, design and implementation of the slot booking logic, and functional testing of the reservation workflow, corresponding to a 100% contribution to that module.

CAPSTONE PROJECT OUTCOME MAPPING SHEET

To be completed by the department/faculty assessor.

| Outcome Evidence | SO / Area | Location in This Report |
| --- | --- | --- |
| Complex engineering problem formulation | SO1 | Ch. 1, Ch. 2 |
| Engineering design under constraints | SO2 | Ch. 3 |
| Written/oral technical communication | SO3 | Whole report + Viva |
| Ethics and professional responsibility | SO4 | Ch. 3 (3.7) |
| Teamwork and leadership | SO5 | Ch. 4 (4.8) |
| Experimentation, analysis, engineering judgment | SO6 | Ch. 4 (4.3–4.6) |
| Independent acquisition of new knowledge | SO7 | Ch. 5 (5.3) |
| Standards | SO2 | Ch. 3 (3.2) |
| Sustainability and societal/environmental impact | SO2/SO4 | Ch. 3 (3.7) |
| Risk assessment and mitigation | SO2/SO4 | Ch. 3 (3.7) |
| Security considerations | Relevant SO | Ch. 3 (3.7) |
| Integrated/system-level engineering | SO1/SO2 | Ch. 3 (3.5) |
| Project planning and management | SO5 | Ch. 4 (4.8) |
| Individual contribution | SO1–SO7 | Contribution Statement + Ch. 4 (4.8) |